Tesla’s Computer Vision pipeline:

Summary:

Tesla’s Autopilot and Full Self-Driving features are built using end to end computer vision pipelines that use multiple cameras and neural networks for semantic understanding and prediction. It combines the concept of both AI and computer vision connected to sensors that rely on the concept of vision of human driver’s.

Below are the stages for Tesla’s CV driving pipeline:

- Multi camera video capture and AI

It is used to understand the environment around the vehicle for driver’s safety. Instead of completely relying on the lidar sensor and radar, it completely focuses on Artificial Intelligence and Computer Vision combined together.

Modern Tesla car contains 8 cameras connected to all sides of the car in order to capture 360 degrees of the surrounding. It includes 3 cameras connected facing forward with different focal lengths (narrow, main and wide), side cameras are used to capture the side views while additional cameras at the back/fender is used to capture the blind spots and the traffic present behind the vehicle where the visible is blocked for the driver.

Each camera delivers a high resolution in real time footage to the system which are custom designed by Tesla ‘s engineers using the concept of neural network which covers the tasks such as object detection, lane recognition, depth analysis and predictions.

By combining the data from multiple cameras, the AI system can be used to detect the depth to know the vehicles present around the car to avoid collision and to take precaution. This helps the driver to maintain a safe distance from the other cars present around it. Also, as there are multiple areas connected for data acquisition if somehow one camera is failed the others will do the task for the safety of the driver.

- Low-level image processing and synchronization

Before ingesting the data to the deep learning architecture which consists of neural networks that are interconnected to each other via neurons, they perform several pre-processing steps to make the videos for the real time analysis process. These steps are very crucial as the model is completely depends on the data, so if the data ingestion is good the decision made by the model will be more precise and accurate.

The first step involves the correction of lens distortion as camera hardware contains lenses which can capture noise that can distort the captured images. After that it adjusts the color, contrast, brightness etc. using different techniques like histogram equalization, color normalization and exposure adjustment to make the image looks consistent even under different lighting condition such as low or high exposure, shadows, sunlight and nighttime.

The second part to calibrate the images captured from all the cameras and synchronize them together. The system analyzes each images and align with the time frame to make the consistency of the images. This part is very important because in autonomous vehicle each frames are linked with each other which then combined together to perform a decision. If any one of the frame got misplaced than sever accidents may happen.

This synchronization helps the model to build a complete 3d structure of the of the environment, track the object around it and to predict the availability of distance need to maintain any collision for the safety of driver.

- Neural networks for analysis

After the images is properly processed and synchronized, it is then sent to the neural network for inference purpose. Each and every cameras present in Tesla are having a different architecture of neural networks based on their different purpose, that analyze the system what each particular camera is seeing. The objective of these networks is to find out hidden information and to extract important features from the images to understand the environment in order to make accurate decisions.

The first task in neural network is image segmentation in which the system classify or identify different object presents in an image. It goes through each and every pixel and then assign a label for it such as road, sidewalk, pedestrian, traffic lights etc. This help the model to identify the areas which is safe for the vehicle to move.

The system also perform object detection to identify important objects present in the image and to draw a bounding box around the classified object. The object can contains of various things for an example cyclist, traffic light, pedestrian etc. By identifying the objects present around the car, it react accordingly.

Tesla uses the architecture of HydraNet for the analysis of the videos captured by the cameras. It first extract the important features from the image after that the layer is split down into sub-branches where each branch focused on different purpose. For example: one branch can be used to identify traffic lights, one to identify the other vehicles, one to identify the persons and many more. This allow parallel processing at a single point of time.

- Multi camera feature fusion and Bird’s‑Eye View

Each camera present in Tesla is used to analyze the image conditions and make predictions. Each camera are assigned different task and has different architecture which they follows. However, relying on individual cameras can be biased so they combined the result of all the cameras through a process called Multi Camera Fusion, which is a projection of 3D structure of the environment.

In this process the data from all the image is collected and than Tesla transform into a 2D Birds Eye View representation. This is similar to seeing a map in google maps. In this top to down view the user will be able to see the objects present around the vehicles. However there are some limitation of 2D Bird Eye View:

- As it mainly capture the ground plane it is unable to focus on capturing the height.

- It also fails when there is a object which is hiding behind an another object also known as occlusion.

To overcome this Tesla has implemented another advanced technique called occupany network, which provides a representation of 3D structures to able to learn the projection from all the axis instead of relying on a flat 2D contour representation of map. To idea is create multiple 3d regions called Voxel.

- Semantic understanding: lane, road and objects

After the extraction of 3d structure from the images captures by multiple cameras, the system has to be given the clear context for all the objects present in an image. This is called as semantic understanding where the relationship is taken into the picture between the objects to understand the deeper context of the image.

Here, different nodes captures the different object present in the environment such as

- Vehcile, pedestrican, cyclists

- Traffic lights and signs

- Lane marking and road edges

- Drivable lane or space

These information are than redirected to the system to take necessary decisions. The system also build a graph from the information to understand the road structure and to understand how it should move in the road.

- Prediction of future movements

Understanding the cars behavious is very important in any autonomous vehicle but we have to also consider the possibility and the assumptions of another vehicles present in the road and to also track the motion of different objects present in the road. Based on these situation the model need to adapt its decision correctly.

Tesla uses Prediction network, it analyzes the situation present in real time and than create a bag of possibilities based upon which than the system finally make the decision. This network study the motion of objects around it to make decisions, it calculate the depth of the object to calculate the approximate distance from the car to the object and than react accordingly.

If there is a moving vehicle which goes behind an object and than re-appears after a while, this also been captured by the system. The prediction model usually works by creating a bunch of possibilities for every object. For example if a vehicle is moving at a very high speed than it slows down again it faster its speed, these all transformation in speed is been assigned some probabilities and based on current situation it chooses the most favorable outcome for making decisions.

It follows 5 main criteria:

1.Purpose of predicting networks

2. Input used for prediction

3. Types of objects predicted

4. Multi trajectory prediction

5. Handling Occlusions

- Trajectory planning and Decision making

After Tesla understands the environment and also the behavious of other vehicles present around it, whether they will move fast or they will move slow, the next step is planning. This stage is used to decide whether the car should go fast or the car should go slow and to improvise the decision accordingly to get the best accurate path which the car should follow.

This stage take all the information from the past stage like the information about the objects present, the motion of the object around it, the lane structure etc. Using these data the system decides multiple possibilities for the vehicle like whether the vehicle should move fast or slow, or did the vehicle need to take a turn or not etc. It follows the neural network decision, here the model is already been trained on huge amount of real-life scenarios dataset and than the demo version is been tested on many situations and than adapted accordingly. Again if somehow a new situation arises it will try to find a way by evaluating all the data that its been trained on. So, instead of hard coding and pre-defining rules for all the situation the model usually adapt it as per the situation, this help the model grows more and eventually the model will become stronger and stronger.

At every situation the model generates 3 major possible paths or trajectories:

- Safety – Here the model avoid any collision with another vehicles, person, pedestrian, trees etc

- Comfort – This ensure that the vehicle should follow a linear speec while acceleration and de-acceleration, which avoid sudden brakes and sudden increase in pace.

- Traffic rule – The model strictly follows the traffic rule by gathering the real time data of the traffic lights. It ensures all the traffic rule should be follows.

Once the system selects the best path the speed is kept constant for a while and then again it paces up or pace down as per situation. Also, the user have the access to limit the cars speed by sampling selecting the top speed from the setting. This has continuously been captured and revised by Tesla to follow a certain path with no sudden change in speed.

- Control and Vehicle actuation

After the best trajectory is selected the command then sent to the control section of the car. This step is used to control the decision of the car like applying brakes, acceleration, switching gears etc.

When the system decides the path and the target speed, it automatically improvised the car acceleration to reach the destination.

It also takes response from multiple sensors like wheel encoders, inertial measurements units and other sensors to take more accurate decisions. It consists of information about the car speed, orientation and movement.

Also, it consist of feature of feedback which is continuously being evaluated if the feedback comes as negative it re-calibrates the training data to avoid any future accidents. This is how the Tesla’s system adapts itself. This feedback is evaluated using a loss function which is than try to reduce ensuring stable and safe driving.

- Data Collection and Training

The major key strength of Tesla is that it learns from it pass mistakes to make the future much easy. For an example if the system is going through a situation where it performs some errors in term so of misjudgment than that particular clip where the decision of Tesla fails has been sent to the company. They analyze the entire data manually and then again fix that bug by applying knowledge into it and then again re-train their main algorithm into it with their advance GPU called Dojo Supercomputer.

After this on next update it again sends the updated patch to again re-calibrate the system entirely to fix that error. This creates a continuous improvement cycle, where the car collects the data on real time basis, the error is considered, the error is being fixed, and the updated models is again sent it to the users. This is how Tesla learns and makes the decision of the model more robust and accurate.